Troubleshooting process of Customer problem of Wireless Connection

AIntroduction:

C.E 3.1 In this career episode will discuss the wireless troubleshooting process that I performed remotely and get a customer a suitable solution on this. I will explain the detail work flow that I followed to get the solution of Data network.

Chronology: April 25, 2023 to April 26, 2023

Company: ADN telecom

Designation: Assistant Engineer, Customer Care Department (CCD)

Location: Dhaka, Bangladesh

B Background:

C.E 3.2 ADN Telecom Ltd. (Advanced Data Network) is one of the most well known ISP (Internet Service Provider) including MPLS, IPLC and IP Telephony service provider in Bangladesh. It is a nationwide network and it coverage the network by Fiber, wireless, satellite, and MPLS services. In this project I am working as an Assistant Engineer along with 1st level engineers for network monitoring and diagnosis the customer issues. Basically, I monitor Infrastructure and Data services (Only Bank customers). This project is to solve the customers wireless connectivity problem of Data service, where I involved from begin to till to the end.

My responsibility is to diagnosis the customer problems and solving of data services of the bank customers. I also take care of critical issues and incidents of different part of the network such as Fiber network, Radio network, logical issues, field operations and Data center. In my department, my main objective is to troubleshoot the primary stage over the phone, if the problem not solved, I involved Fiber team or Radio team to perform for field support at the POP location or Customer location which is depends on the critical incident and also depends on the primary troubleshooting (power status, device status and device situation), and aim to solve the customer problems within 24 hours.

On the other hand, I also work for Infrastructures. I monitor all POP over the Bangladesh including Base, PDB, and POP switches including ring topology from Solar winds software. I also monitor Upstream from the MRTG (Multi Router traffic Grapher).

All engineers used Zoho Software to create Ticket and take all customers complain, troubleshooting process and final root cause.

C.E 3.3 The Project objectives are:

- Understanding of customer technical problem.

- To detect the fault of radio connectivity problem.

- To analyze the problem from Base station or POP sectors in Radio base software (Cambium)

- Communicate with customer for troubleshooting in initial stage to findings the actual fault.

- Work with Radio team for troubleshooting radio hardware or software work.

- Work with NOC team for logical part.

- Work with regional team to assign the team at fault area.

- Managing customers and sharing real time updates of technical work.

C.E 3.4 The hierarchy of the company:

Level 1 (Assistant Engineer)

Level2 (Internet)

Level 1 (Assistant Engineer)

Level 2 (Bank)

Level 1 (Assistant Engineer)

Level2 (Enterprise)

Supervisor

Head of the Department

CTO (Acting)

C Personal Engineering Activities

C.E 3.5 I gain knowledge about Radio network from Radio department training. I also have some knowledge about wireless network from my Bachelor’s degree. My bachelor’s degree was ECE [Electronics and Communication Engineering], where I learned about Wireless communication system, computer networks, high speed telecommunication, network security, data communication, antenna & propagation, digital signal processing and microwave engineering, etc. In addition, I got training about the radio connectivity checking process from Cambium software. Basically, I am working remotely with radio team along with customer. In this project, my responsibility was to diagnosis the customer problem. I was working remotely to check radio all network connection; I also was work with radio team and NOC team to solve the problem. I have had five assigned customers, these five customerswere using Data connectivity, and also, I was taking care Infrastructure of whole Bangladesh. I gathered knowledge from CCNA course to check network path from POP router to customer end router. My radio team gave me training to understand the radio technical terms and I learned about OSI model to understand the Data connectivity in my bachelor’s graduation degree. From all experience, it helps me to understand customer problem and diagnosis their problem along with my radio team, NOC team and regional team.

C.E 3.6 In this episode, I work for Bank (Customer) who is using wireless network and also, they use data (Layer 2) connectivity.The Bank name is Rupali bank and their branch was Budhhata Shatkhira.

The problem I got from customer 25 April 2023. Client didn’t get our connectivity properly. I took complained from whatsapp. The customer complained that their Budhhata branch was getting down. I called to Budhhata branch customer to understand their problem. After taking their technical term and device status of their problem, I checked Radio connectivity including customer Data connectivity

C.E 3.7I checked radio connectivity of customer. First, I checked Radio link. I ping radio IP 10.225.150.234. I got link down and sometimes I got destination host unreachable.

I called to customer and took device status.The radio device was unplugged because they couldn’t get service from our connectivity. After connected the device still I was getting down. Therefore, I rebooted the radio device by customer and getting same problem. Again I tried to check cables and router situation. Customer did cable reconnection from radio device side and from router side, after that, the situation was same. The primary remote troubleshooting was done but the link was not up. Therefore, I check thoroughly from source (ADN router) to destination (Customer Budhhata Bank end).

C.E 3.8 The customer radio connectivity was connected from ADN Ashasuni Sector-1. This sector was connected from ADN Ashasuni POP. Therefore, I checked POP from Solarwinds. As I am also monitoring the infrastructure, I checked POP Ashasuni POP which was up and also I checked the ADN Ashasuni Sector-1 which was also up.Therefore, I understood that the customer connection was not down for POP or sector. I checked to customer destination in many ways. First, I checked from ADN Ashasuni POP loss and latency. Because a huge loss or latency may be the reason for customer's service interruptions. So, I did ping 10.232.13.190 to check POP, the uplink of this pop is Summit. Therefore, I telnet IP: 10.111.111.18 to entry to check Ashashuni POP’s loss and latency.

>Telnet 10.111.111.18

Cisco router>>

User name: ****

Password: *****

Summit-AGG-RT-1#ping 10.232.13.190 size 500 r 500

Type escape sequence to abort.

Sending 500, 500-byte ICMP Echos to 10.232.13.190, timeout is 2 seconds:

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

!!!!!!!!!!

Success rate is 100 percent (500/500), round-trip min/avg/max = 4/7/12 ms

After checking POP I couldn’t find any loss or latency, therefore, I checked sector-1 which connected was from POP’s switch. There are three sectors was added in POP. In these sectors, each sector has three customers who are connected and they all were using wireless connection.The sectors are belonging to Cambium Base. Cambium network is the most common base station that our company uses for customer wireless connectivity. Because Cambium is user friendly software. Besides, I also can check Nano base where I learned from Radio team members.

C.E 3.9 I checked sector 1 where Rupali Bank Budhhata branch customer was exists. Before check the customer connection, I checked from Cambium Network NMS software to find the customer availability. I went to Assasuni POP, then sector 1 and then I got Rupali Bank Budhhata Bazar link was showing offline in the Dashboard in Cambium network NMS. It shows the link was down for 1 hour and 56 m. In this NMS I can see customer Bandwidth, frequency, LAN connectivity, etc. The below picture is NMS of Cambium Network, which was use for monitoring purpose and where we can check all POP and POP sectors, including customer link. I can see Rupali bank Buddhata Branch was getting offline.

C.E 3.10 Similarly, I checked another way, I checked from Assausuni sector-1. I browse http://10.225.150.226/ to login to Cambium networks of Assassuni sector. I checked Customer list availability from this Base sector. First I went to wireless and then from monitor I found two customers were available but I couldn’t find Rupali bank Buddhata connection, where I mentioned before same as when I checked Cambium monitor. Below picture showed the

C.E 3.11 After troubleshooting and checking all details I informed to Radio team. Radio team was also communicating with customer to make sure his all connection stability. Though the radio connection wasn’t up, they tried to troubleshoot from POP side remotely. But it was not work, and I talked with customer again to check their radio adapter connection. Customer changed another vendor’s radio adapter and then I found the radio link still was not up. Hence, the radio connection was down, I informed to our Radio team and shared my findings. Therefore, the Radio team has sent to the customer and in the mean while I gave update to the customer and took a access arrangement for the troubleshooting work.

C.E 3.12On the next day, our concern regional team was assigned and our radio team was aligning for the work. Regional engineer and his team were visit at the customer location. They changed the adapter which I mentioned them, and then I was communicating to regional engineer for getting update. Meanwhile, I checked whether the radio connection service was available or not. After changing the adapter still the Radio connection was not in live. I communicate with the Radio team and we decided to check the Radio device which was on the tower and the tower was on the rooftop of the building.

Our team has checked the radio device and got the device was off, the team tested the device and it was not working. So, Radio team engineer informed me regarding this and we decided to replace the new Radio device.

C.E 3.13After changing the Radio device, radio team setup all configurations. I took help from Radio team to check radio connection thoroughly. Therefore, radio link was up. I checked more, to ensure the wireless service was okay.

I checked Radio IP 10.225.150.234

ADN>ping 10.225.150.234 -t

Pinging 10.225.150.234 with 32 bytes of data:

Reply from 10.225.150.234: bytes=32 time=21ms TTL=57

Reply from 10.225.150.234: bytes=32 time=26ms TTL=57

Reply from 10.225.150.234: bytes=32 time=22ms TTL=57

Reply from 10.225.150.234: bytes=32 time=20ms TTL=57

Reply from 10.225.150.234: bytes=32 time=87ms TTL=57

Reply from 10.225.150.234: bytes=32 time=22ms TTL=57

Reply from 10.225.150.234: bytes=32 time=23ms TTL=57

Ping statistics for 10.225.150.234:

Packets: Sent = 7, Received = 7, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 20ms, Maximum = 87ms, Average = 31ms

C.E 3.14After getting Radio connection I took confirmation from customer but customer still was not getting service. Therefore, I checked from Cambium software to find the issue, I checked Radio details from Rupali bank BudhhataRadio Cambium software IP in the bowser http://10.225.150.226/. I checked Ethernet connection, which was 100Mbps was running, that means customer LAN connection was okay. Then I checked the throughput for checkingcustomer’sBandwidth usage, but I did not get anybandwidth from it.

However, I checked customer service IP: 10.168.142.242. I checked from Rupali Bank Head Office IP 10.168.202.10 with source IP 10.168.141.1, where I found the service status wasgetting no route to host. This ping report is shown below-

[support@Rupali Bank DC Motijheel] > ping 10.168.142.242 src-address=10.168.141.1

SEQ HOST                                     SIZE TTL TIME  STATUS

0                                                         no route to host

1                                                         no route to host

2                                                         no route to host

3                                                         no route to host

4                                                         no route to host

5                                                         no route to host

6                                                         no route to host

7                                                         no route to host

8                                                         no route to host

9                                                         no route to host

10                                                         no route to host

sent=11 received=0 packet-loss=100%

C.E 3.15The Radio connection was up but due to some logical issue rose; I took help from NOC department to solve this problem. I informed to NOC engineer who was checking the issue and he solved that problem. It was routing issue that NOC engineer was fixed that problem. After fixing it, finally I got the link was up by checking service IP: 10.18.142.242.

In addition, I checked all process one by one for smooth wireless network customer connectivity. First, I checked from POP Assausuni sector- 1 IP: 10.225.150.226, I browse this sector Radio IP: 10.225.150.226 to login Cambium software. In this sector, I got Rupali bank Budhhata customer was in the list which was not listed when link was down. To check this list I went to Monitor> Wireless and then the MAC address was shown registered for Rupali bank Budhhata.

After that, I checked Customer Radio IP: 10.225.150.234 which was also getting smooth connection. To check further details, I browse 10.225.150.234 to login customer radio Cambium software; I check Ethernet Status was 100Mbps running, then I checked bandwidth from Monitor>throughput Chart. I also checked network details (MAC address, duplex mode and interface) from Cambium software.

C.E 3.16 Next, I Checked customer Bandwidth from MRTG (Multi Router traffic Grapher), which shown that customer already using data connection.

I checked service IP: 10.168.142.242 as a same process which I did before. The ping report has given below to understanding better.

@Rupali Bank DC Motijheel] >ping  10.168.142.242 src-address=10.168.141.1

SEQ HOST                                     SIZE TTL TIME  STATUS

0 10.168.142.242                             56 250 22ms

1 10.168.142.242                             56 250 22ms

2 10.168.142.242                             56 250 24ms

3 10.168.142.242                             56 250 25ms

4 10.168.142.242                             56 250 22ms

5 10.168.142.242                             56 250 22ms

6 10.168.142.242                             56 250 22ms

7 10.168.142.242                             56 250 23ms

8 10.168.142.242                             56 250 22ms

9 10.168.142.242                             56 250 23ms

10 10.168.142.242                             56 250 21ms

11 10.168.142.242                             56 250 22ms

12 10.168.142.242                             56 250 24ms

13 10.168.142.242                             56 250 20ms

14 10.168.142.242                             56 250 24ms

15 10.168.142.242                             56 250 20ms

16 10.168.142.242                             56 250 22ms

17 10.168.142.242                             56 250 22ms

18 10.168.142.242                             56 250 22ms

19 10.168.142.242                             56 250 22ms

sent=20 received=20 packet-loss=0% min-rtt=20ms avg-rtt=22ms max-rtt=25ms

Therefore, I communicate with my Radio team and Regional Engineer to share RFO (Reason for Outage) and also, I learned from NOC department engineer that how the logical issue was solved.  I took all documents in Zoho. I took every update and procedure inZoho, I also gave update with ETR (Estimated Time for Restoration) to the client to manage the. I communicate to Rupali Bank HQ to take confirmation the service. I also communicate with Rupali bank Budhhata branch customer to take confirmation of wireless network smooth connectivity. After getting confirmation I shared link up details with root cause to customer by email and whatsapp.

D Summery

C.E 3.17This episode gave me more knowledge about wireless connection. I understand more to troubleshoot of radio connection. From the beginning of the customer complain, I troubleshoot radio connection in a primary stage to understand the actual fault. After that, I took help from Radio team, regional team and NOC team to solve this issue. I thank